Batch Analysis Pipeline for 4-Channel TIFF Images

Overview

This MATLAB pipeline performs batch analysis of four-channel TIFF images (e.g., STED or confocal datasets).
For each selected image, the script generates a comprehensive set of visualization figures and quantitative overlap coefficients, and exports all numerical results into a single CSV file for downstream statistical analysis.

The pipeline is fully automated, requires no manual ROI selection, and is designed to ensure reproducibility and consistency across large datasets.

Input Requirements

- Multi-page TIFF files with at least four image planes

- Channel assignment:

  - Channel 1: Red

  - Channel 2: Green

  - Channel 3: Blue

  - Channel 4: Gray (displayed as intensity added to RGB channels)

All images are assumed to be aligned and of identical dimensions across channels.

How to Run the Analysis

- Open MATLAB and add the script directory to the MATLAB path.

- Run the main batch function:

- batch_analyze_4ch_tiffs2

- A file selection dialog will appear.
Select one or multiple 4-channel TIFF files.

- The script will process each file sequentially and report progress in the MATLAB Command Window.

Each image is analyzed independently; if one file fails, the remaining files will continue to be processed.

Output Summary

For each TIFF file, the following outputs are generated:

Figures (saved as .fig files)

- Fig1–Fig3: Combined into a single main figure

- Fig4: Pairwise channel overlap analysis

- Fig5: Triple-channel overlap analysis

Figures are saved in the same directory as the input TIFF file, using the original filename as a prefix.

Numerical Results

- All overlap coefficients are aggregated into a table

- A timestamped CSV file is saved in the same directory

- The results table is also assigned to the MATLAB base workspace as:

- batch_overlap_results

Figure Descriptions

Fig1 — Original Normalized Images

Each channel is independently normalized to the range 01using linear scaling.
Merged RGB images are generated to visualize pairwise and multi-channel combinations.

Purpose

- To inspect raw intensity distributions

- To provide a reference view independent of contrast enhancement or thresholding

Fig2 — Contrast-Enhanced (LUT-Applied) Images

A percentile-based LUT is applied to each channel:

- Intensities below the 90th percentile are mapped to black

- Intensities at or above the 99.9th percentile are mapped to white

- Intermediate values are linearly scaled

The original 16-bit dynamic range is preserved during processing before display.

Purpose

- To enhance weak structures

- To approximate manual LUT adjustments commonly performed in ImageJ/Fiji

- To standardize contrast across datasets

Fig3 — Otsu Thresholded Binary Images

Otsu’s method is applied to the LUT-enhanced images to generate binary masks for each channel.
Merged pseudocolor images illustrate spatial overlap of thresholded signals.

The fourth (gray) channel is displayed as a white intensity overlay.

Purpose

- To visualize automatic thresholding results

- To compare intensity-based and structure-based representations

- To assess the upper bound of detectable overlap

[Figure 3 placeholder]

Fig4 — Pairwise Overlap Analysis

All six pairwise channel combinations are analyzed:

- ch1–ch2

- ch1–ch3

- ch2–ch3

- ch1–ch4

- ch2–ch4

- ch3–ch4

For each pair, a row of panels displays:

- Merged LUT-enhanced image

- Normalized channel A

- Normalized channel B

- Pixel-wise overlap map with overlap coefficient

Purpose

- To visualize spatial overlap

- To quantify pairwise colocalization using a consistent definition

Fig5 — Triple-Channel Overlap Analysis

All four triple-channel combinations are analyzed:

- ch1–ch2–ch3

- ch1–ch2–ch4

- ch1–ch3–ch4

- ch2–ch3–ch4

Overlap is computed as the pixel-wise minimum intensity across three normalized channels.

Each row displays:

- Merged LUT-enhanced image
2–4. Normalized individual channels

- Triple-overlap map with overlap coefficient

Purpose

- To quantify higher-order colocalization

- To identify regions where three molecular components co-exist

Definition of Overlap Coefficients

For each channel X, intensities are normalized such that:

∑X=1

Pairwise overlap

overlap(X,Y)=∑min⁡(X,Y)

Triple overlap

overlap(X,Y,Z)=∑min⁡(X,Y,Z)

This definition captures intensity-weighted spatial co-occurrence, analogous to Manders-type metrics but extended to multi-channel comparisons.

CSV Output Format

Each row corresponds to a single TIFF file.
Columns include:

- coef12, coef13, coef23, coef14, coef24, coef34

- coef123, coef124, coef134, coef234

Missing values are filled with NaN to ensure consistent table structure.

Notes on Reproducibility

- No manual thresholding or ROI selection is used

- All processing steps are deterministic

- Visualization and quantification are derived from the same underlying data

- The pipeline is suitable for batch-level statistical comparisons across experimental conditions

Typical Use Cases

- STED or super-resolution colocalization analysis

- Multi-protein synaptic organization studies

- High-throughput comparison of molecular overlap across genotypes or treatments